APPROVED

General designer of OJSC “MVZ im. M.L. Milya” A.G. Samusenko

24 December 2009

BULLETIN № T2775-БЭ-АБ

Helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (MI-171) and their modifications.

Subject: life time and service life of helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (MI-171) and their modifications.

This bulletin applies to helicopters of type Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (Mi-171) and their modifications, that are located outside the Russian Federation, the owners of which are aviation companies of foreign countries, including countries of the CIS.

By this bulletin all aviation companies that maintain helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (MI-171) and their modification helicopters in civil variant are notified hereby about the newly established time and service life:

- Assigned life time 12000 hours;

- Assigned service life 30 years – for helicopters Mi-17, Mi-8MTV-1 issued before 1991;

- Assigned service life 25 years – for helicopters issued after 1991;

- Life time before first repair 2000 hours;

- Service life before first repair 9 years;

- Overhaul life 2000 hours;

- Overhaul service life 8 years.

Maintenance of helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (MI-171) should be conducted in accordance with the assigned maintenance publications.

Parts and components of helicopters included in the attachment to this bulletin, as well as those parts not included in the list that have a life time and/or service life before first repair, overhaul or assigned service life of more than that assigned for the helicopter, are to be maintained within their assigned life time and service time.

Parts and components of helicopters not included in the attachment to this bulletin (whose life time and service life, before first repair and overhaul life, are less than that assigned for the helicopter as indicated above), are permitted to be maintained before the end of the period of life time or service life before first repair or overhaul life of the helicopter on which they are installed. Maintenance of indicated parts and components beyond their assigned life time and service life, before the end of the life time or service life assigned to that helicopter before repair, is permitted on the assumption that these parts and assemblies were installed on the helicopter when it was produced or during a capital repair.

Items without passports are to be maintained within life time and service life of the helicopter on which they are installed.

According to the specialists of OJSC “MVZ im M.L. Milya, ” technical estimation of specific helicopter extension of its assigned life time and service life is possible. Actions on the extension of life time and service life of the helicopter are executed on a contract basis.

ANNOTATION:

- Engines TB3-117BM, TB3-117MT, BK2500-03, AI-9B, SAFIR 5K/G MI and main gearbox BP-14 are to be maintained according to life time and service time given in their logbooks.

- In case of regular utilization of the helicopter (more than 10 % from acting overhaul life) for operations that differ from the operations specified in the “Flight Manual”, for example: transporting trees, erecting towers, fire bucket operations and other operations that are characterized with short flight duration and high repeatability of transient states:

- Helicopter should be equipped with flight recorder BUR-1-2 (BUR-1-2-Zh), weight control device, vibration damper, and ;

- Logbook should contain records of the running hours in the configurations indicated above.

Bulletin № T2133-БЭ-АБ ”Helicopters Mi-8MTV, Mi-17, Mi-17-1B, Mi-17B5, Mi-172, Mi-8AMT, Mi-17”, expires on the subject of life time and service life of the helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (Mi-171).

ATTACHMENT:

List of components of helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (Mi-171) with indicated life time and service life as of the date of the bulletin approval.

Deputy of General designer

OJSC “MVZ im. M.L. Milya”				 			A.M. Nikishov

Attachment to the Bulletin T2775-БЭ-АБ

List of parts and components of helicopters Mi-17, Mi-8MTV-1 (Mi-172), Mi-8AMT (Mi-171) with indicated life time and service life, acting for them on the date of the bulletin approval.

| № | Name of part (device) | Code of device | Life time (hours), sevice life (years) of helicopters of types: |  |  |  |  |  |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Mi-17, Mi-8MTV-1, Mi-172 |  |  |  |  |  | Mi-8AMT, Mi-171 |  |  |  |  |  |  |
|  |  |  | Before 1st repair |  | Overhaul repair |  | Assigned life |  | Before 1st repair |  | Overhaul repair |  | Assigned life |  |  |
|  |  |  | hours | years | hours | years | hours | years | hours | years | hours | years | hours | years |  |
| 1 | Main rotor blades: -Made by KVZ. - -Made by U-UAZ. - Change only the set of the same production. | 8AT-2710-00 8AT-2710-00 | - | - | - |  | 2000 2000 | 7 7 | - - |  |  |  | 2000 2000 | 7 7 | Service life for diff. Climatic zones – according to the passport. |
| 2 | Main rotor hub | 8-1930-00 ser.2 | 1500 | 7 | 1500 | 7 | 3000 | - | 1500 | 7 | 1500 | 7 | 3000 | - |  |
| 3 | Vibration damper | 8Mt-1280-100 8AMT-1250-00 | 1500 - | 6 7 | 1000 - | 6 7 | 2500 2000 | 24 | 1500 - | 6 7 | 1000 - | 6 7 | 2500 2000 | 24 | Made by KVZ. Made by U-UAZ. |
| 4 | Pitch control mechanism | 8-1950-000 | 1500 | 8 | 1500 | 8 | 3000 | - | 1500 | 8 | 1500 | 8 | 3000 | - |  |
| 5 | Intermediate gearbox | 8A-1515-000 8M-1515-000 | 2000 3000 | 10 15 | 2000 3000 | 8 15 | 6000 12000 | - | 2000 3000 | 10 15 | 2000 3000 | 8 15 | 6000 12000 | - | Assigned service life is not limited. |
| 6 | Tail rotor gearbox | 246-1517-000 8M-1517-000 | 2000 3000 | 10 15 | 2000 3000 | 8 15 | 6000 12000 | - | 2000 3000 | 10 15 | 2000 3000 | 8 15 | 6000 12000 | - |  |
| 7 | Drive shaft | 8A-1516-000 8M-1516-000 | 2000 3000 | 10 14 | 2000 3000 | 8 14 | 6000 12000 | - | 2000 3000 | 10 14 | 2000 3000 | 8 14 | 6000 12000 | - |  |
| 8 | Fan | 8A-6311-00 Ser.4 | 2000 | 8 | 1500 | 8 | 6500 | - | 2000 | 8 | 1500 | 8 | 6500 | - |  |
| 9 | Fan drive shaft | 8A-6314-000 8AT.6314.00 | 1500 1500 | 8 8 | 1500 1500 | 8 8 | 3000 3000 | - | 1500 1500 | 8 8 | 1500 1500 | 8 8 | 3000 3000 | - |  |
| 10 | Tail rotor: - Tail rotor hub, - Tail rotor blades. | 246-3904-000 246-3914-000 246-3925-000 | 1000 1000 | 7 7 | 1000 1000 - | 7 7 | 3000 3000 1000 | - 7 | 1000 1000 - | 7 7 | 1000 1000 - | 7 7 | 3000 3000 1000 | -* -* 7 | *Assigned service life is not limited. |
| 11 | Hydraulic booster system | 8MT-5104-200 172.5104.000 8MT-5104-200 8AMT-5104-200 | 1500 1500 | * * | 1500 1500 | * * | 5500 5500 | * * | 1500 1500 | * * | 1500 1500 | * * | 5500 5500 | * * | *Service life as airframe. |
| 12 | Tail rotor control circuit | Pr-15, 875-2300-1 2-5200-02 TU | - |  | - |  | 1000 | 7 | - |  |  | - | 1000 | 7 |  |
| 13 | Shock damper of main chassis | 8A-4101-00B 1 8A-4101-00B-2 | 4000 4000 | 10 10 | 4000 4000 | 8 8 | 26000 26000 | 27 27 | 4000 4000 | 10 10 | 4000 4000 | 8 8 | 26000 26000 | 27 27 |  |
| 14 | Lever damper strut of front chassis | 8A-4201-00A | 4000 | 10 | 4000 | 8 | 26000 | 27 | 4000 | 10 | 4000 | 8 | 26000 | 27 |  |
| 15 | Hydraulic booster | KAU-30B, PA-60B, KAU-115a, KAU-115AM | 1500 1500 1500 | 10 10 10 | 1500 1500 1500 | 8 8 10 | 7500 7500 6000 | 30 30 30 | 1500 1500 1500 | 10 10 10 | 1500 1500 1500 | 8 8 10 | 7500 7500 6000 | 30 30 30 |  |
| 16 | Diaphragm of hydraulic accumulator | 15-5303-10-6 | - |  | - |  | - | 8 | - |  | - |  | - | 8 |  |
| 17 | Service tank | 8AT-6101-00 8AMT-6101-00 | - |  | - |  | - | 21 | - |  | - |  | - | 21 | Including time of transportation and storage. |
| 18 | Hoses of oil system of engines and main gearbox(oil B-3V), fuel and air system | RMB TU38 105419-90; 3U, 1U TU38005-1515-93 | - |  |  |  |  | 10 |  |  |  |  |  | 10 | Including 8 years of maintenance. |
| 19 | Hoses: - Hydraulic system, filling hydr. boosters and exiting from pump NSh-39M to hydraulic panel; -Hydraulic system that drain oil from hydr. boosters and inlet to pump NSh-39M; | Type of hose: 22P10-240 TU38005-1515-85(92),21PB-150, dev.80 TU38005-1515-85(92) | - |  |  |  |  | 10 |  |  |  |  |  | 10 | Service life – from the date of hose production, marked on the hose. During repair – checked. |
|  |  | Fluoroplastic 8DO.447.005 8DO.447.021 | - |  |  |  | 3000 3000 | 25 25 |  |  |  |  |  |  |  |
|  | - Hydraulic system other than indicated above. | 5Y6-100, 5Y20-85 | - |  | - |  | - | 10 10 | - |  | - |  | - | 10 10 | Service life – from the date of hose production, marked on the hose. During repair – checked |
| 20 | Gyrohorizon | AGB-3K ser.3 | 3000 | 7 | 3000 | 5 | 9000 | 15 |  |  |  |  |  |  |  |
|  |  | AGB-96D,AGB-96D-S,AGB-96R,AGB-96R-S |  |  |  |  |  |  | 1500 | 10 | 1500 | 5 | 4500 | 20 |  |
|  |  | AGR-74-15 ser.2, AGK-77-15 |  |  |  |  |  |  | 1000 1000 | 10 10 | 1000 1000 | 8 8 | 3000 3000 | 20 20 |  |
| 21 | Generators | CGC-40PU GT40PC8V | 1500 | 9 | 1500 | 8 | 7000 | 30 | 2000 | 10 | 1500 | 10 | 7000 | 20 |  |
| 22 | External additional fuel tanks | VDB.6130.200 777-6150-00 | - 2000 | - 9 | - 2000 | - 8 | 1500 12000 | 25 30 | - 2000 | - 9 | - 2000 | - 8 | 1500 12000 | 25 30 |  |

NOTES:

- Given in the list; information about life time and service life of parts and components is reference data. Information about changes is given to the interested organizations in the established order.

- Not given in this list; assigned life time and service life periods of devices are not regulated while following all the requirements of storage and maintenance according to original maintenance documentation.

- Extension of life time and service life before first repair or overhaul repair of individual parts and components (numbers №1-8, 10, 11, 13, 14 of the list) is possible according to the specialists’ of OJSC “MVZ im. M.L. Milya” technical estimation.